ElRobot 7+1 DOF

Dataset & Pretrained Model Research Report

NormaCore Platform  ·  Raspberry Pi 5  ·  Pick-and-Place Optimisation

Research date: June 2026  ·  Sources: HuggingFace, GitHub, ArXiv, Tavily, Apify

# 1. Executive Summary

Goal: Achieve reliable pick-and-place on NormaCore ElRobot (7+1 DOF, Raspberry Pi 5) in the shortest possible time. Strategy: SmolVLA pretrained base + SO-101 community datasets + 50 ElRobot demonstrations. Training on cloud GPU (free Colab T4/A100). Inference via action-chunked deployment on RPi5.

Key findings from cross-ecosystem research (HuggingFace, GitHub, ArXiv, OpenPI, DROID, Open X-Embodiment):

• ElRobot is confirmed to use STS3215 servos — the same as SO-100/SO-101. LeRobot driver adaptation requires adding only one extra joint dimension (SO-101: 6-DOF action space; ElRobot: 7+1 DOF = 8-dim action space).

• SmolVLA (450M params) is the optimal pretrained model: fine-tuned on SO-101 pick-place, 29 community variants, MIT-adjacent license, Colab-trainable, action-chunked RPi5 inference feasible at ~2s/chunk.

• ACT (52M params) is the fastest inference option for RPi5 (182ms on CPU) and requires only 50-100 demos. Best fallback if SmolVLA RPi5 latency is unacceptable.

• 16,000+ LeRobot datasets are publicly available on HuggingFace (as of Sep 2025). SO-101 has 3,965 datasets and 58,299 episodes. Community standard is 50 episodes per task.

• π0 / π0.5 require dedicated GPU at inference (not RPi5-compatible). OpenVLA-7B requires 15-25GB VRAM. Both are ruled out for hackathon deployment.

# 2. Assumptions

The following are stated assumptions governing this research:

A1. Hardware: Raspberry Pi 5 is the inference device. Training occurs off-device (cloud GPU or workstation).

A2. ElRobot is NOT yet officially supported by LeRobot library. LeRobotDepot confirms STS3215 servo compatibility, so SO-101 driver adaptation is the integration path.

A3. Action space: ElRobot has 7 rotational joints + 1 gripper = 8-dimensional joint-space action. SO-101 has 6-dimensional action. One extra joint dimension must be handled when using SO-101 datasets.

A4. Task scope: Primary goal is a working pick-and-place demo, not multi-task generalisation.

A5. Demo collection: The operator can use leader-follower teleoperation (STS3215 is leader-compatible) to collect 50 demonstrations.

A6. Camera: At least one RGB camera (top or wrist) is available. SmolVLA was trained with 'up' and 'side' views; matching these reduces fine-tuning data requirements.

A7. Internet access is available for HuggingFace dataset streaming and model download.

# 3. Critical Technical Note: Action Space Mismatch

IMPORTANT: All SO-101 datasets use a 6-dimensional action space (5 joints + 1 gripper). ElRobot has 7 joints + 1 gripper = 8-dimensional action space. Direct loading of SO-101 datasets for ElRobot fine-tuning will FAIL unless the action space is remapped. Options: (A) Mask the extra joint and fix it at a neutral angle during fine-tuning. (B) Collect all 50 demos natively on ElRobot — recommended for hackathon reliability.

# 4. Top 10 Pretrained Models — Ranked for ElRobot

Ranked by ElRobot compatibility, RPi5 deployability, fine-tuning ease, and data availability.

| # | Model | Params | License | Trained On | Source / URL | RPi5 Inference | Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SmolVLA (lerobot/smolvla_base) | 450M | MIT | LeRobot community datasets — SO-100, SO-101, pick-place, diverse HF datasets | huggingface.co/lerobot/smolvla_base | ~2s/chunk (action chunking), FEASIBLE | 9/10 ★★★★★★★★★☆ |
| 2 | ACT — Action Chunking Transformers | 52M | MIT (LeRobot) | ALOHA bimanual (50 demos/task) | github.com/huggingface/lerobot | 182ms CPU — EXCELLENT for RPi5 | 8/10 ★★★★★★★★☆☆ |
| 3 | SmolVLA (HuggingFaceTB/smolvla_libero) | 450M | MIT | LIBERO simulation pick-place, Panda | huggingface.co/HuggingFaceVLA/smolvla_libero | ~2s/chunk (action chunking), FEASIBLE | 8/10 ★★★★★★★★☆☆ |
| 4 | π0.5 (pi0.5 via LeRobot) | 3.5B | Apache 2.0 | 400h mobile manipulation + static robots, diverse home environments | huggingface.co/lerobot/pi0.5 | NOT feasible RPi5 — GPU required | 7/10 ★★★★★★★☆☆☆ |
| 5 | π0 (pi0 via openpi / LeRobot) | 3.5B | Apache 2.0 | 7 robot platforms, 68 tasks + full OXE dataset | github.com/Physical-Intelligence/openpi | NOT feasible RPi5 — 16GB+ GPU required | 6/10 ★★★★★★☆☆☆☆ |
| 6 | OpenVLA-OFT (Stanford OFT recipe) | 7B | MIT | 970K episodes — Open X-Embodiment (70+ domains) | openvla-oft.github.io / github.com/openvla | NOT feasible RPi5 — 16-25GB VRAM | 6/10 ★★★★★★☆☆☆☆ |
| 7 | OpenVLA-7B (base) | 7B | MIT | Open X-Embodiment 970K robot trajectories | huggingface.co/openvla/openvla-7b | NOT feasible RPi5 — ~15GB bfloat16 | 5/10 ★★★★★☆☆☆☆☆ |
| 8 | Diffusion Policy (LeRobot) | 263M | MIT (LeRobot) | ALOHA, PushT, various manipulation | github.com/huggingface/lerobot | 3454ms CPU — too slow for reactive control | 5/10 ★★★★★☆☆☆☆☆ |
| 9 | NVIDIA GR00T N1.5 (via LeRobot 0.4.0) | ~1B | NVIDIA open model | Humanoid + manipulation, large-scale NVIDIA data | huggingface.co/nvidia/GR00T-N1.5 | NOT feasible RPi5 — Jetson Orin recommended | 4/10 ★★★★☆☆☆☆☆☆ |
| 10 | Octo (cross-embodiment foundation) | ~93M | Apache 2.0 | OXE subset, 800K trajectories, multi-robot | github.com/octo-models/octo | Possible RPi5 (small model) but poor task specificity | 4/10 ★★★★☆☆☆☆☆☆ |

## 4.1 Inference Latency Comparison (ms)

Source: LeRobot paper (arXiv:2602.22818) and community benchmarks. CPU = standard desktop. MPS = Apple Silicon.

| Model | Params | CPU (ms) | MPS (ms) | RTX 4090 (ms) | RPi5 Feasibility |
| --- | --- | --- | --- | --- | --- |
| ACT | 52M | 182 | 43 | 5 | ✅ Excellent — 182ms |
| SmolVLA | 450M | 2028 | 722 | 99 | ⚠️ Feasible with action chunking |
| Diffusion Policy | 263M | 3454 | 370 | 614 | ❌ Too slow for reactive |
| π0 / π0.5 | 3.5B | N/A (OOM) | N/A (OOM) | 209 / 569 | ❌ GPU required |
| OpenVLA-7B | 7B | N/A (OOM) | N/A (OOM) | N/A | ❌ 15GB+ VRAM only |

# 5. Top 10 Datasets — Ranked for ElRobot Fine-Tuning

Ranked by pick-and-place relevance, transferability to ElRobot 7+1 DOF, and fine-tuning value.

| # | Dataset | Episodes | Robot | Tasks | Format / URL | ElRobot Fit | FT Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | lerobot/svla_so101_pickplace | 50 | SO-101 | Pick-place (cube in bowl), 30fps, 2 cameras (up+side) | HF: lerobot/svla_so101_pickplace ¦ LeRobot v2 | 8 — same servo, 6→8 DOF remap/10 ★★★★★★★★☆☆ | 9/10 ★★★★★★★★★☆ |
| 2 | xinjiehu76/so101-pick-place-dataset | ~50 | SO-101 | Red cube → gray bowl, 2 camera views, teleoperated | HF: xinjiehu76/so101-pick-place-dataset ¦ LeRobot | 8 — same servo, 6→8 DOF remap/10 ★★★★★★★★☆☆ | 8/10 ★★★★★★★★☆☆ |
| 3 | ud-smart-city/lerobot-so-101-manipulations | 20 | SO-101 | Pick-and-place + basic manipulation, multi-view (front/wrist/top) | HF: ud-smart-city/lerobot-so-101-manipulations | 7 — small but clean/10 ★★★★★★★☆☆☆ | 7/10 ★★★★★★★☆☆☆ |
| 4 | lerobot SO-101 community pool (3,965 datasets / 58K episodes) | 58,299 total | SO-101 | Wide variety — pick, place, sort, stack, push | HF filter: robot_type=so101 ¦ LeRobot format | 7 — large pool, variable quality/10 ★★★★★★★☆☆☆ | 8/10 ★★★★★★★★☆☆ |
| 5 | lerobot SO-100 community pool (5,161 datasets / 78K episodes) | 78,510 total | SO-100 | Similar tasks to SO-101 but older driver | HF filter: robot_type=so100 ¦ LeRobot format | 7 — same servos as ElRobot/10 ★★★★★★★☆☆☆ | 7/10 ★★★★★★★☆☆☆ |
| 6 | DROID (Large-scale in-the-wild) | 76,000 | Franka (7-DOF) | Diverse manipulation, 22 robot variants, language-annotated | droid-dataset.github.io ¦ HF + RLDS ¦ updated Apr 2025 | 6 — 7-DOF match, different servo/kinematics/10 ★★★★★★☆☆☆☆ | 6/10 ★★★★★★☆☆☆☆ |
| 7 | Open X-Embodiment (OXE) | 1,000,000+ | 22 robots (Franka, WidowX, xArm…) | 527 skills, pick-place dominant, 60 labs worldwide | github.com/google-deepmind/open_x_embodiment ¦ RLDS TFRecord | 5 — diverse but needs RLDS→LeRobot conversion/10 ★★★★★☆☆☆☆☆ | 5/10 ★★★★★☆☆☆☆☆ |
| 8 | BridgeData V2 | 60,000 | WidowX (6-DOF) | 60+ tasks, tabletop manipulation in kitchen/office settings | rail.eecs.berkeley.edu/datasets/bridge_release ¦ RLDS | 5 — 6-DOF, different kinematics/10 ★★★★★☆☆☆☆☆ | 5/10 ★★★★★☆☆☆☆☆ |
| 9 | RoboMIND (107K trajectories) | 107,000 | Franka, UR5e, humanoid | 479 tasks, 96 objects, RGB-D multi-view, standardised collection | HF: x-humanoid-robomind/RoboMIND ¦ RLDS/LeRobot | 4 — large scale, heavy conversion effort/10 ★★★★☆☆☆☆☆☆ | 4/10 ★★★★☆☆☆☆☆☆ |
| 10 | ExylosAi/pick_and_place_sample | ~20 | Generic arm (VR-captured) | Pick-and-place, visual domain randomisation, LeRobot-native | HF: ExylosAi/pick_and_place_sample ¦ LeRobot ¦ Apache 2.0 | 5 — generic but LeRobot-native, lightweight/10 ★★★★★☆☆☆☆☆ | 4/10 ★★★★☆☆☆☆☆☆ |

# 6. Kinematic Similarity to ElRobot (7+1 DOF)

ElRobot: 7 rotational joints + 1 gripper, STS3215 servos, tabletop, 3D-printed. URDF + MuJoCo sim provided.

| Robot | DOF | Gripper DOF | Servo/Actuator | Data Volume (LeRobot) | Kinematic Similarity | Similarity Rank |
| --- | --- | --- | --- | --- | --- | --- |
| SO-ARM107 (ajinkyagorad) | 6+1 | Yes | STS3215 (identical) | Limited — experimental | Very High — same servo family, 1 joint fewer | ★★★★★ 1st |
| SO-101 (TheRobotStudio / HF) | 5+1 (6-DOF action) | Yes | STS3215 (identical) | 3,965 datasets / 58K eps | High — same servos, 2 fewer joints | ★★★★☆ 2nd |
| SO-100 (TheRobotStudio / HF) | 5+1 (6-DOF action) | Yes | STS3215 (identical) | 5,161 datasets / 78K eps | High — same servos, older version | ★★★★☆ 3rd |
| Koch v1.1 | 5+1 | Yes | STS3215 / custom | 849 datasets / 21K eps | Medium-high — similar size, different link lengths | ★★★☆☆ 4th |
| DROID robot (Franka-based) | 7 (7-DOF EE) | Parallel gripper | Franka FCI (proprietary) | 76,000 episodes | Medium — 7 DOF match, but different actuators | ★★★☆☆ 5th |
| xArm 6/7 (Ufactory) | 6 or 7 | Various | Brushless + harmonic | 450K eps (OXE) | Medium — 7-DOF variant close kinematically | ★★★☆☆ 5th |
| WidowX 250 (Trossen) | 6-DOF | Parallel | Dynamixel | 60K eps (BridgeData) | Lower — 6 DOF, Dynamixel, different link lengths | ★★☆☆☆ 7th |
| ALOHA (bimanual) | 2x 6-DOF | Yes (bimanual) | Dynamixel | Large ALOHA dataset | Low — bimanual, different form factor | ★★☆☆☆ 8th |

# 7. Fine-Tuning Strategy Analysis

Evidence-based comparison of five fine-tuning approaches. Research basis: ACT paper (Zhao et al. 2023), SmolVLA blog, ggando.com field report, roboticscenter.ai dataset guide, LeRobot paper.

| Option | Approach | ElRobot Demos | Pros | Cons / Risk |
| --- | --- | --- | --- | --- |
| A — Zero-shot | Pretrained model only (SmolVLA_base) | 0 | No data collection needed. Fast to start. | Will fail — ElRobot not in pretraining. Generalisation does not extend to new embodiments without fine-tuning. |
| B — Minimal FT ✅ RECOMMENDED | SmolVLA_base + 20-30 ElRobot demos | 20-30 | Very fast collection (~2h). Low overhead. Sufficient for constrained pick-place demo. Community validated at ~20 eps. | Lower success rate (~50-60%). Limited generalisation. Workspace must be tightly controlled. |
| C — Standard FT ✅ RECOMMENDED | SmolVLA_base + 50 ElRobot demos | 50 | Community standard. SmolVLA SO-101 FT benchmark uses 50 eps. ACT trained at 50 eps hits 80%+ success. Best risk/reward. | ~4-6h collection time. Needs GPU training (~4h on Colab L4/T4). Config fixes required (n_action_steps=50). |
| D — Extended FT | SmolVLA_base + 100 demos | 100 | Higher generalisation. More robust to workspace variance. Better recovery from near-misses. | ~8-12h collection. Training ~8h on Colab. Diminishing returns for single-task demo. Overkill for hackathon. |
| E — From scratch | ACT/Diffusion Policy, no pretrain | 200+ | No pretraining overhead. Full control over architecture. | Requires 200+ demos minimum for any reliability. No semantic generalisation. NOT recommended. |

# 8. Hackathon Deployment Path — Raspberry Pi 5 + NormaCore

Objective: Highest probability of a working pick-and-place demo in minimum time. Hardware: RPi5 + ElRobot + 1-2 USB cameras + NormaCore MCP integration.

## 8.1 Recommended Architecture

| Component | Choice | Notes |
| --- | --- | --- |
| Training (off-device) | Google Colab (free T4/A100) or local RTX GPU | Cloud / workstation — NOT on RPi5 |
| Policy | SmolVLA fine-tuned on 50 ElRobot demos | Primary recommendation |
| Fallback Policy | ACT fine-tuned on 50 ElRobot demos | 182ms CPU inference — ideal if SmolVLA too slow |
| Inference execution | Action chunking (chunk_size=30, ~3s of motion) | RPi5 handles 2s inference then executes chunk |
| Integration | LeRobot Robot class with custom ElRobot driver (STS3215) | ~1 day integration effort |
| Camera | 2x USB RGB (640x480 @ 30fps) — top + side views | Match SmolVLA's pretraining camera layout |
| Teleoperation | Leader-follower with second ElRobot arm (same servos) | For demo collection only |

## 8.2 Step-by-Step Implementation Timeline

| Time | Phase | Action |
| --- | --- | --- |
| Day 1 AM | Hardware setup | Register ElRobot as SO-101-compatible device in LeRobot. Test motor comms (STS3215 bus). Verify all 7+1 joints are responsive. Add 8th joint to action config. |
| Day 1 PM | Camera integration + teleoperation | Mount top + side cameras. Calibrate. Verify leader-follower teleoperation works end-to-end. Test RPi5 camera stream. |
| Day 2 AM | Dataset collection (50 episodes) | Collect 50 pick-and-place demos. Use LeRobot record script. Verify episode quality using lerobot replay. Target: cube pick from fixed position, place in fixed bowl. |
| Day 2 PM | Fine-tune SmolVLA on Colab | Upload dataset to HuggingFace Hub. Run SmolVLA fine-tuning on Colab T4 (~4h). Fix config.json (n_action_steps=50). Push fine-tuned model to HF Hub. |
| Day 3 AM | Deploy + validate | Download fine-tuned model to RPi5. Run inference with action chunking. Evaluate 10 rollouts. Measure success rate. |
| Day 3 PM | Tune or fallback | If success rate < 50%: collect 20 more demos and retrain. If latency issue: switch to ACT (52M, 182ms CPU). If generalisation needed: adjust workspace lighting/position. |

# 9. Final Recommendation

| Category | Recommendation |
| --- | --- |
| Best pretrained model | lerobot/smolvla_base (SmolVLA, 450M) — Trained on SO-101 pick-place community data. 29 community fine-tuned variants. Colab-trainable. Action-chunked RPi5 deployment feasible. |
| Primary fine-tuning dataset | lerobot/svla_so101_pickplace — 50 episodes, 6-DOF SO-101, cube pick-place. Remap 6→8 DOF by masking joint 6 (neutral) OR collect native ElRobot demos directly. |
| Recommended fine-tuning strategy | Option C: SmolVLA_base + 50 native ElRobot demonstrations. This is the community-validated standard (50-episode cluster in LeRobot ecosystem). Achieves 70-85% success on constrained tasks. |
| Fallback model (latency-critical) | ACT (LeRobot) — 52M params, 182ms CPU inference. If SmolVLA 2s/chunk is too jerky for the demo, ACT trained on 50-100 demos will give smoother real-time control on RPi5. |
| Transfer learning strategy | Pretrained model (SmolVLA_base) + SO-101 pick-place community data as auxiliary pretraining context + 50 ElRobot native demos for fine-tuning. DO NOT train from scratch. |
| ElRobot demos required | Minimum 20 (for constrained demo), Recommended 50 (for reliable demo), Extended 100 (for generalisation). Each episode takes ~2-3 minutes to collect. |
| Expected time to first success | Realistic: 72-96 hours end-to-end (hardware setup → data collection → training → deployment). Optimistic: 48 hours with prior LeRobot experience and pre-assembled hardware. |
| Expected success rate (50 demos) | SmolVLA: 65-80% success on in-distribution pick-place. ACT: 75-90% on constrained workspace. Both degrade significantly outside training workspace boundary. |

# 10. Risks & Mitigations

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| RPi5 inference latency too high for smooth demo (SmolVLA ~2s/chunk) | High | Medium | Switch to ACT (182ms CPU). Use action chunking with chunk_size=30 so robot moves continuously during inference window. |
| Action space mismatch (6-DOF SO-101 data vs 8-DOF ElRobot) | High | High | Collect ALL 50 demos natively on ElRobot. Avoids remapping complexity entirely. |
| ElRobot LeRobot integration time overrun | Medium | High | Use SO-101 driver directly, change only joint count in config. STS3215 bus is identical. Budget 1 day for integration. |
| Low success rate after 50 demos | Medium | High | Tighten workspace constraints. Ensure consistent lighting. Add 20 more demos. Switch to ACT if SmolVLA generalisation is the bottleneck. |
| Model overfits to training positions | High (known issue) | Medium | Vary starting cube position within ±5cm during collection. Community reports: SmolVLA fails outside ±10cm workspace boundary. |
| Colab training session timeout | Medium | Low | Use save_freq=500 steps. Push checkpoints to HF Hub incrementally. Use Colab Pro or download intermediate checkpoint. |

# 11. Key Sources

| Resource | URL |
| --- | --- |
| SmolVLA paper & HF blog | huggingface.co/blog/smolvla |
| SmolVLA base model | huggingface.co/lerobot/smolvla_base |
| SO-101 pick-place dataset | huggingface.co/datasets/lerobot/svla_so101_pickplace |
| SmolVLA SO-101 fine-tuning field report | ggando.com/blog/smolvla-so101 |
| LeRobot paper (arXiv) | arxiv.org/html/2602.22818v1 |
| LeRobot GitHub | github.com/huggingface/lerobot |
| OpenVLA GitHub | github.com/openvla/openvla |
| OpenVLA-OFT project | openvla-oft.github.io |
| π0 open-source announcement | pi.website/blog/openpi |
| openpi GitHub (π0 / π0-FAST) | github.com/Physical-Intelligence/openpi |
| DROID dataset | droid-dataset.github.io |
| Open X-Embodiment | github.com/google-deepmind/open_x_embodiment |
| RoboMIND | huggingface.co/datasets/x-humanoid-robomind/RoboMIND |
| LeRobotDepot (ElRobot entry) | github.com/maximilienroberti/lerobotdepot |
| ElRobot technical overview | industrialmonitordirect.com/blogs/knowledgebase/elrobot-71-dof-3d-printed-robotic-arm-technical-overview |
| NormaCore GitHub | github.com/norma-core/norma-core |
| SmolVLA fine-tuning guide | docs.phospho.ai/learn/train-smolvla |
| LeRobot datasets October 2025 stats | kamenski.me/articles/lerobot-datasets-oct-2025 |
| VLA inference on XPUs (arXiv 2604.24447) | arxiv.org/html/2604.24447v1 |
| ACT policy explained 2026 | roboticscenter.ai/en/blog/act-policy-explained |

Report generated by Claude (Anthropic) using Tavily search, Apify RAG web browser, and direct HuggingFace / GitHub / ArXiv research. Research date: June 2026.